le90m_wrap.cpp

[ Python Module : le90m ]

LE90M_BS = 0

[ Constant: int  ]

LE90M_GATE = 1

[ Constant: int  ]

LE90M_ADDR = 2

[ Constant: int  ]

LE90M_PARAM = 3

[ Constant: int  ]

LE90M_DISABLE = 0

[ Constant: int  ]

LE90M_TSSCOPE = 1

[ Constant: int  ]

LE90M_TBSGATE = 2

[ Constant: int  ]

LE90M_TLED = 3

[ Constant: int  ]

LE90M_TGATEES = 4

[ Constant: int  ]

LE90M_TINHIBIT = 5

[ Constant: int  ]

LE90M_TESSTART = 6

[ Constant: int  ]

LE90M_FLED = 7

[ Constant: int  ]

LE90M_FTEST = 8

[ Constant: int  ]

all times are in 0.1 sec units in BCD format for example 0x16 =

1.6 seconds frequencies are specified as a divisor from base frequency

all functions are using already converted values

1.  class LE90MDATA

===================

[ created from struct LE90MDATA ]

LE90MDATA_tbs_set(LE90MDATA *,short )

LE90MDATA_tbs_get(LE90MDATA *)

[ Member data: returns short  ]

time between start and begin spill

LE90MDATA_tgate_set(LE90MDATA *,short )

LE90MDATA_tgate_get(LE90MDATA *)

[ Member data: returns short  ]

width of the gate

LE90MDATA_disable_set(LE90MDATA *,short )

LE90MDATA_disable_get(LE90MDATA *)

[ Member data: returns short  ]

disable manual programming

LE90MDATA_tsscope_set(LE90MDATA *,short )

LE90MDATA_tsscope_get(LE90MDATA *)

[ Member data: returns short  ]

time between start and scope start

LE90MDATA_tbsgate_set(LE90MDATA *,short )

LE90MDATA_tbsgate_get(LE90MDATA *)

[ Member data: returns short  ]

time between begin spill and gate

LE90MDATA_tled_set(LE90MDATA *,short )

LE90MDATA_tled_get(LE90MDATA *)

[ Member data: returns short  ]

width of led pulses window

LE90MDATA_tgatees_set(LE90MDATA *,short )

LE90MDATA_tgatees_get(LE90MDATA *)

[ Member data: returns short  ]

time between end of gate and end spill

LE90MDATA_tinihibit_set(LE90MDATA *,short )

LE90MDATA_tinihibit_get(LE90MDATA *)

[ Member data: returns short  ]

width of inihibit signal after end spill

LE90MDATA_tesstart_set(LE90MDATA *,short )

LE90MDATA_tesstart_get(LE90MDATA *)

[ Member data: returns short  ]

time between end spill and next start

LE90MDATA_fled_set(LE90MDATA *,short )

LE90MDATA_fled_get(LE90MDATA *)

[ Member data: returns short  ]

led pulses frequency divisor, 1=100Hz, 0=none

LE90MDATA_ftest_set(LE90MDATA *,short )

LE90MDATA_ftest_get(LE90MDATA *)

[ Member data: returns short  ]

test pulses frequency divisor, 1=100kHz, 0=none

2.  class LE90M

===============

[ created from class LE90M ]

new_LE90M(host,number)

[ Constructor: returns LE90M * ]

delete_LE90M(LE90M *)

[ Destructor: returns void  ]

LE90M_bcd2dec(LE90M *,w)

[ Member : returns short  ]

LE90M_dec2bcd(LE90M *,w)

[ Member : returns short  ]

LE90M_tbs(LE90M *)

[ Member : returns short  ]

LE90M_tgate(LE90M *)

[ Member : returns short  ]

LE90M_write(LE90M *,addr,data)

[ Member : returns void  ]

LE90M_read(LE90M *,addr)

[ Member : returns short  ]

LE90M_enable(LE90M *)

[ Member : returns void  ]

LE90M_disable(LE90M *)

[ Member : returns void  ]

LE90M_print(LE90M *,data)

[ Member : returns void  ]